PETROLEUM AND NATURAL GAS REGULATORY BOARD

NOTIFICATION

New Delhi, the 29th December, 2017

F. No. INFRA/PNGRB/T4S/P&PPPL/04/2016.––– In exercise of the powers conferred by section 61 of the Petroleum and Natural Gas Regulatory Board Act, 2006 (19 of 2006), the Petroleum and Natural Gas Regulatory Board hereby makes the following regulations to amend the Petroleum and Natural Gas Regulatory Board (Technical Standards and Specifications including Safety Standards for Petroleum and Petroleum Product Pipelines) Regulations, 2016, namely:-

- Short title and commencement.

- These regulations may be called the Petroleum and Natural Gas Regulatory Board (Technical Standards and Specifications including Safety Standards for Petroleum and Petroleum Product Pipelines) Amendment Regulations, 2017.

- They shall come into force on the date of their publication in the Official Gazette.

- In the Petroleum and Natural Gas Regulatory Board (Technical Standards and Specifications including Safety Standards for Petroleum and Petroleum Product Pipelines) Regulations, 2016, -

- Clause 3.5.7.4 shall be substituted, namely :-

“All stud, bolts and nuts used in petroleum and petroleum products pipelines should be as per ASTM A 194 / A193.”;

- Clause 3.7.1.2 shall be omitted;

- Clause 4.10.1.2 shall be substituted, namely :-

“No welding (other than welds required for tie-in, installation of mainline valves, hot tapping and appurtenances) and/or mechanical handling of pipelines is permitted after pressure testing. The integrity of these welds shall meet the requirements as specified in API-1104.”;

- Clause 4.10.2.1 (iv) shall be substituted, namely :-

“Mainline valves along with branch pipe should be pressure tested before installation.”;

- Clause 6.12.1.5 (ii) shall be substituted, namely :-

“By installing full encirclement welded split sleeves or leak clamps to contain internal pressure and shall have a design pressure of not less than the maximum allowable operating pressure. This shall be fully welded both circumferentially and longitudinally. These repairs shall conform to the requirements as specified in ASME 31.4.”.;

- Clause 7.9.3 shall be renumbered as Clause 7.9.2.1

- Clause 7.9.4 shall be renumbered as Clause 7.9.2.2.

VANDANA SHARMA, Secy.

[ADVT.III/4/Exty./377/18]

Foot Note: Principal regulations were notified vide No. G.S.R. Infra/ T4S/ P&PPPL/01 /2014., dated 12.02.2016.